Multi-Voltage Macro Core: RTL to GDSII (Vol. 1)

32nm Multi-Voltage ASIC Design Reference

(0.95V / 0.75V Dual Supply Physical Design Methodology)

The Complete Implementation Handbook

Author: Ajay Mallesh | Aviraj Dhale | Selva Prakash

Reviewer: Gemini Pro 3.1 & Ajay Mallesh

Project Contributors: Aviraj Dhale | Selva Prakash | Venkatesh Vattem | Praveen

Version: Version 2.0 (2026)

Date: 14/07/2026

“This project is generated with Gemini Pro 3.1 and crafted by Ajay Mallesh with ❤️.”

Project Contributors:

Aviraj Dhale | Selva Prakash | Venkatesh Vattem | Praveen |

whose contributions across development, testing, and feature enhancements helped shape this project.

Technology Focus

| Parameter | Specification |
| --- | --- |
| 1. Technology Node | 32nm CMOS (SAED32) |
| 2. Design Type | Multi-Voltage ASIC |
| 3. Core Type | Macro Engine (mv_macro_engine) |
| 4. Instance Count | 76K Instances |
| 5. Macro Count & Type | 100 Hard Macros (16 lp_ram_subsystem SRAMs + 84 Default) |
| 6. Voltage Domains | 0.95V (VDD) / 0.75V (VDDL) |
| 7. Flow Style | RTL → GDSII |
| 8. Primary Tools | Synopsys (Design Compiler, ICC2, PrimeTime) |
| 9. Timing Methodology | MCMM (Multi-Corner Multi-Mode) |
| 10. Signoff Style | PrimeTime / StarRC |
| 11. Power Strategy | Multi-Domain Mesh PDN (with VDDL Guard Bands) |
| 12. Clocking | H-Tree CTS + NDR (Double Spacing) |
| 13. Target Audience | Physical Design Engineers / ASIC Architects |

Copyright Notice:

© 2026 Ajay Mallesh. This project is open-source and released for educational and   non-commercial community use. You are free to use, modify and distribute this project in accordance with the project license.

Author: Ajay Mallesh

Email: ajaymalavalli912@gmail.com

LinkedIn: AjayMallesh

Project Contributors:

LinkedIn: Aviraj Dhale

LinkedIn: Selva Prakash

LinkedIn: Venkatesh Vattem

Legal Disclaimer

The methodologies, TCL examples, UPF constructs, synthesis flows, timing constraints and physical implementation strategies described in this document are intended exclusively for educational and research purposes.

Tool commands and flows may vary depending on:

- tool version,

- process node,

- foundry NDA constraints,

- technology LEF/DEF structure,

- extraction deck version,

- signoff correlation methodology,

- and EDA vendor implementation changes.

The author assumes no responsibility for:

- silicon failure,

- timing closure loss,

- LVS/DRC violations,

- power collapse,

- EM degradation,

- tape-out delay,

- or fabrication loss arising from improper application of the concepts described herein.

Contents

Project Introduction & Overview	8

Chapter 1: Files and Directories (Linux)	9

Chapter 2: RTL Implementation	9

Chapter 3: Synthesis	10

Chapter 4: Floorplan	13

Chapter 5: Port Placements	14

Chapter 7: Macro Placements	16

Chapter 8: Power Delivery Network	18

Chapter 9: Congestion checks	20

Chapter 10: Placement	21

Chapter 11: Clock Tree Synthesis	24

Chapter 12: Routing	27

Chapter 13: Signoff	29

Chapter 14: Sign-off Timing Analysis & ECO Flow	30

Chapter 15: Advanced IR Drop	33

Chapter 16: Hard Macro Conversion (Block Abstraction)	35

Chapter 17: Formal Verification (Logic Equivalence Checking)	38

Chapter 18: Design for Manufacturing (DFM)	39

Chapter 19: The ECO Flow & Final Handoff	40

# Project Introduction & Overview

Multi-Voltage Macro Engine Architecture

The mv_macro_engine project is a high-performance multi-voltage ASIC design implemented in 32nm CMOS technology. It features a robust architecture comprising 76K standard cells and 100 hard macros, including 16 lp_ram_subsystem SRAMs and 84 default hard macros.

To optimize for power efficiency and meet strict SoC thermal constraints, the design employs a dual-voltage architecture based on aggressive voltage scaling. The logic is partitioned into two distinct domains:

- High-Performance Domain (0.95V / VDD): This domain houses the primary computational engine and high-speed logic. It operates at 0.95V to ensure maximum throughput and timing closure, utilizing the saed32rvt_ss0p95v125c.db library.

- Low-Power Domain (0.75V / VDDL): This domain incorporates the memory subsystems and low-speed control logic. By operating at 0.75V, the design significantly minimizes both dynamic switching power and static leakage. It utilizes the saed32rvt_ss0p75v125c.db library, optimized for low-voltage operation.

Physical Design Challenges

The multi-voltage architecture of the mv_macro_engine introduces several sophisticated backend design requirements:

- UPF-based Power Intent Integration: Seamless integration of logical power intent with the physical design database.

- Level Shifter Management: Ensuring signal integrity for all crossings between the 0.75V and 0.95V domains using dedicated ulvl and dlvl level shifters.

- Macro-Centric Floorplanning: Strategic placement of 100 hard macros, including the memory subsystems, while maintaining routability and minimizing wire detours.

- Dedicated Voltage Areas & PDN: Implementation of a multi-domain mesh Power Delivery Network (PDN) featuring VDDL guard bands for the low-power engine.

- H-Tree Clock Distribution: Managing an H-Tree clock topology to minimize skew while handling the voltage-level crossings at the clock spine.

- MCMM Timing Analysis: Balancing timing closure across complex multi-corner multi-mode scenarios to ensure robustness across process, voltage, and temperature variations.

Tools

- Synthesis: Synopsys Design Compiler

- Physical Design: Synopsys ICC2

- Signoff: Synopsys PrimeTime / StarRC

### Chapter 1: Files and Directories (Linux)

1.1. Directory creations

- Command: mkdir mv_macro_engine

- cd mv_macro_engine/

1.2. Files creations

- Command: gvim <file_name>

- cat <file_name>

- nano <file_name>

1.3. Automated Script

- source setup_env.tcl

*** This script will create all the required directories

### Chapter 2: RTL Implementation

2.1 RTL File Implementation Logic

For a multi-voltage design, the RTL logic must be cleanly partitioned into distinct hierarchical blocks based on their target voltage. In this mv_macro_engine design, the architecture is split into two main sections:

- 1. The High-Performance Control Block: Runs on the main 0.95V (VDD) supply. This handles the fast I/O interfaces, main state machines, and clock generation.

- 2. The Low-Power Macro Engine: Runs on the scaled 0.75V (VDDL) supply. This block performs the heavy, latency-tolerant computation and utilizes the lp_ram_subsystem and other SRAM hard macros.

The top-level Verilog file (mv_macro_engine_top_sfo.v) acts as a wrapper that wires these blocks together.

2.2 Level Shifters Declarations

When a signal travels from the 0.75V (VDDL) domain back to the 0.95V (VDD) domain, a Low-to-High Level Shifter is required. Without it, the 0.75V "High" signal won't fully turn off the 0.95V PMOS transistors, causing a short circuit (crowbar current).

- How they are declared: In modern physical design, we do not hardcode level shifters into the RTL.

- The RTL just connects the wires normally.

- Instead, level shifters are declared and automatically inserted using the UPF (Unified Power Format) file during synthesis

### Chapter 3: Synthesis

3.1 What is Synthesis?

Synthesis is the process of translating human-readable RTL code (like Verilog) into a technology-mapped gate-level netlist. The tool reads the behavioral logic and converts it into physical logic gates (AND, OR, Flip-Flops) available in the target 32nm standard cell library, while trying to meet timing and power requirements.

3.1.1 Types of Synthesis

- Logical Synthesis: Traditional synthesis that assumes zero wire delays. It only looks at the logic gates.

- Physical Synthesis (Topographical): Advanced synthesis that predicts physical layout (placement and wire lengths) to give much more accurate timing and power estimates before the actual Physical Design phase.

3.2 Synthesis used in this design

For this mv_macro_engine core, we use Physical (Topographical) Synthesis using Synopsys Design Compiler (DC). Because this is a dual-rail design, the synthesis tool must also be Multi-Voltage Aware, meaning it reads our power intent and automatically inserts the required level shifters between the 0.95V and 0.75V domains.

3.3 Files / Inputs required for Synthesis

To run synthesis, the tool needs five critical inputs:

3.3.1 RTL

The verified Verilog files (e.g., sram_stub.v, sram_stub_pci.v, sram_stub_reg.v, lp_ram_subsystem.v, mv_macro_engine_top_sfo.v) containing the design logic.

3.3.2 .lib

The logic libraries provided by the foundry. Because we have two voltage domains, we must load libraries for both the 0.95V standard cells and the 0.75V (VDDL) standard cells, usually at worst-case timing corners.

3.3.3 UPF

The IEEE 1801 Unified Power Format file (mv_power_intent.upf). This tells the tool where the VDD and VDDL domains are and rules for inserting level shifters.

3.3.4 SDC

Synopsys Design Constraints (mv_constraints.sdc). This file defines the clock frequencies, input/output delays and false paths.

3.3.5 Scanchains

Design for Testability (DFT) setup files. These tell the tool how to stitch the flip-flops together into long "scan chains" so the chip can be tested for manufacturing defects after fabrication.

3.4 Synthesis script file - ./scripts/dc_compile.tcl

A simplified look at the main commands inside the synthesis script:

Tcl (Simplified Example) / Not to execute this alone

# 1. Read the design

analyze -format sverilog {./inputs/sram_stub.v ./inputs/sram_stub_pci.v ./inputs/sram_stub_reg.v ./inputs/lp_ram_subsystem.v ./inputs/mv_macro_engine_top_sfo.v}

elaborate mv_macro_engine_top

# 2. Load constraints and power intent

source ./inputs/mv_constraints.sdc

load_upf ./inputs/mv_power_intent.upf

# 3. Stitch scan chains for DFT

insert_dft

# 4. Compile the design into 32nm gates

compile_ultra -scan -timing_high_effort_script

3.5 Generated output files

Once compile_ultra finishes, it generates the following deliverables for the Physical Design (PnR) team:

3.5.1 Golden Netlist

".v" file containing only 32nm standard cell instances and their wire connections. All abstract RTL is gone.

3.5.2 UPF

An updated ".upf" file. The tool updates the original UPF to include the exact names and locations of the level shifters it inserted.

3.5.3 Scandef

A .def file containing the scan chain trace. This ensures the PnR tool (ICC2) knows how the flip-flops are chained together and can optimize their placement.

3.5.6 SDC (not recommended for Physical Design)

The tool generates a forward-annotated SDC file. Note: It is generally not recommended to use this generated SDC for physical design. It often contains cluttered, auto-generated tool variables. It is best practice to pass the clean, original SDC to the PnR team.

3.6 Reports

Before handing the files off to PnR, these logs must be reviewed:

3.6.1 report_timing

Checks the critical paths. Setup timing should be met (or very close to met) here. Hold timing is generally ignored at this stage because the clock tree doesn't exist yet.

3.6.2 report_area

Checks the total cell count and silicon footprint. We verify that the design is roughly hitting our target of instances.

3.6.3 report_qor

The Quality of Results report. Provides a quick summary of the worst negative slack (WNS), total negative slack (TNS), cell count and leakage power.

3.7 Sanity checks

To sign off on synthesis, two crucial checks must pass clean:

3.7.1 check_design

Scans the netlist for logical errors. You must ensure there are zero floating inputs and zero multi-driven nets (where two outputs try to drive the same wire).

3.7.2 check_mv_design

Scans the multi-voltage architecture. This ensures that every wire crossing from the 0.75V VDDL domain to the 0.95V VDD domain successfully received a level shifter and that there are no illegal power connections.

3.8 Automated Script for synthesis

- source ./scripts/synthesis_sfo.tcl

*** This will do synthesis and generate the output files. Note: change the paths accordingly ***

### Chapter 4: Floorplan

4.1 Creating NDM

Before physical design can begin, the tool needs a unified database containing both logical timing and physical layout information. In Synopsys ICC2, this is the New Data Model (NDM). We create the NDM by merging the synthesized .v netlist, the .db timing libraries, and the physical .lef/FRAM files (including saed32_lp9m_tech.ndm and saed32_sram_lp.ndm) for both the 0.95V and 0.75V domains.

4.2 Initialize Floorplan

Floorplanning establishes the physical silicon canvas for the 76K instances and 100 hard macros. This phase is executed using the automated script located at ./scripts/floorplan/floorplan_script.tcl.

4.2.1 Read Netlist

The tool loads the structural netlist (mv_macro_engine_top_netlist_sfo.v) generated by the synthesis team, mapping the logical instances to physical 32nm standard cell shapes.

4.2.2 Create Core and Die Area

We define the overall chip dimensions. The Die Area includes the outer I/O pad ring, while the Core Area is the internal region where the standard cells and macros are placed. For the mv_macro_engine, we use a rectilinear shape with a 1:1 aspect ratio, a 5-micron core boundary offset, and a strict utilization target of 70% (0.7). This precise 70% utilization ensures enough blank space is left for the 100 hard macros (including the 16 lp_ram_subsystem SRAMs), routing wires, and inserting clock buffers later.

Example:

initialize_floorplan -shape R -side_ratio {1 1} -core_utilization 0.7 -core_offset 5 -site_def unit -use_site_row

4.3 Sanity checks

4.3.1 check_floorplan

Validates that the core area is legally defined, boundaries are closed, and there are no overlapping hard macros that violate placement rules.

### Chapter 5: Port Placements

5.1 Block Pin Constraints - Placing ports

In a hierarchical design like the mv_macro_engine, the external input/output signals must enter and exit the block through physical pins on the core boundary. Pin constraints define exactly which edge (top, bottom, left, right) and which metal layers the ports should use, optimizing the dataflow into the high-speed computational engine and the low-power subsystems.

For this project, you have the flexibility to define your own custom port placements based on your specific top-level integration requirements. Alternatively, you can execute the pre-defined port locations using the provided reference files:

- Port Constraints: ./docs/port locations/ports_locations.tcl

- Execution Script: ./scripts/ macros_and_ports_placement_script/ ports_placement_individual_script.tcl

Note: High-speed clock and critical data ports are restricted to upper metal layers (e.g., M5/M6) to minimize RC delay entering the core.

5.2 Sanity checks

5.2.1 check_pin_placement

Confirms that all pins have been successfully dropped onto the core boundary and align properly with the 32nm routing grid.

5.2.2 check_pre_pin_placement

A preventative check run before physical placement. It verifies that your pin constraint file is syntactically valid and that you aren't trying to place pins on an illegal metal layer or a highly congested edge (such as regions blocked by the SRAM macros).

Chapter 6: Multi-voltage Design

6.1 UPF

We read the .upf file outputted by Synthesis into ICC2

(load_upf ./outputs/fixed_high_fanouts/mv_macro_engine_top.upf). This ensures the physical tool understands exactly which cells belong to the high-performance domain and which belong to the low-power domain. We also manually set the voltages (set_voltage) to prevent UPF checker errors (UPF-057) before proceeding.

6.2 create voltage area

We physically draw the PD_LP region on the floorplan canvas. This optimized boundary creation is executed using the script located at ./scripts/voltage_area_efficent/v_a_efficient_mv_macro_engine.

The tool will strictly place the 0.75V mv_macro_engine standard cells inside this designated boundary. Critically, all 16 lp_ram_subsystem SRAM hard macros are physically constrained to reside entirely within this low-voltage domain, completely separated from the 0.95V logic.

6.3 commit UPF

This command tells ICC2 to finalize the power intent. It maps the logical UPF definitions to the physical voltage area we just drew and creates the actual power nets (VDD_DEFAULT and VDD_LP) in the database.

6.4 Sanity checks

6.4.1 check_mv_design

Validates that the physical floorplan correctly supports the multi-voltage intent without illegal overlaps.

6.4.2 report_mv_cells

Generates a list of all level shifters and isolation cells, confirming they were successfully carried over from synthesis.

6.4.3 report_mv_path -level_shifter -all_path

Traces every wire crossing from the 0.75V VDD_LP area to the 0.95V VDD_DEFAULT area to guarantee a level shifter physically sits between them, preventing crowbar currents.

### Chapter 7: Macro Placements

7.1 Macro Placement Strategy

With the floorplan initialized, ports placed, and the multi-voltage areas (UPF) successfully defined, the next critical step is placing the hard macros. Macro placement dictates the routability and timing success of the entire physical design. Macros generally act as physical blockages and should be placed toward the core boundaries to avoid fracturing the standard cell placement area.

Note: It is not strictly necessary to use the predefined macro placements provided in this reference kit. You have the flexibility to place the macros according to your own custom dataflow requirements, logical flylines, and top-level SoC constraints.

7.2 Execution Scripts and Location Files

If you choose to use the verified reference floorplan to guarantee routability for the mv_macro_engine, the precise X/Y coordinates and orientations for all 100 macros (16 lp_ram_subsystem SRAMs and 84 default macros) are defined and executed using the following files:

- Macro Locations File: ./docs/macro_locations/macro_locations.tcl

- Execution Script: ./scripts/macros_and_ports_placement_script/macro_placement_script.tcl

7.3 Domain-Specific Macro Constraints

Because this is a multi-voltage design, macro placement must strictly adhere to the physical power domains established in the previous chapter:

- Low-Power SRAMs: The 16 lp_ram_subsystem macros must be placed entirely inside the PD_LP (0.75V) voltage area boundary so they properly connect to the VDD_LP power grid.

- Default Macros: The remaining 84 high-speed macros are placed in the default PD_TOP (0.95V) domain.

- Halos and Blockages: Placement halos (keepout margins) are automatically applied around all macros by the script to prevent standard cells from packing too closely to the macro pins, which reduces local routing congestion.

7.4 Sanity checks

7.4.1 check_legality -macros_only

Validates that there are zero overlapping hard macros, that all macros are legally snapped to the 32nm manufacturing grid, and that their physical orientations are permitted by the foundry rules.

7.4.2 check_mv_design

Run immediately after macro placement to ensure that none of the 16 lp_ram_subsystem macros have accidentally breached the boundaries of the PD_LP voltage area, which would cause severe multi-voltage power domain violations.

### Chapter 8: Power Delivery Network

8.1 Create Tap and bound cells

Before laying metal, we place specific physical-only standard cells:

- Tap Cells: Placed at regular intervals to tie the silicon substrate to ground and n-wells to power, preventing latch-up.

- Boundary Cells (Endcaps): Placed at the end of every standard cell row to protect the delicate logic gates from manufacturing damage.

8.2 Sanity checks

8.2.1 check_boundary_cells

Ensures all rows are properly capped and that the maximum allowed distance between tap cells is not violated.

8.3 creating PG nets

We construct the power grids (VDD, VDDL, and VSS) using upper metal layers to distribute current evenly and prevent IR-drop. The standalone script for this stage is available at: ./scripts/Powerplan/powerplan_mv_macro_engine.tcl.

For the low-power mv_macro_engine region, the implementation prioritizes direct, explicit routing over complex automation. The VDDL mesh is generated ensuring it does not break before the inner guard band; the VDDL shapes must connect continuously edge-to-edge with the guard band to guarantee uncompromised power delivery. Furthermore, standard cell macro ring strategies are explicitly commented out in the reference scripts to strictly maintain this defined topology.

8.4 Sanity checks

8.4.1 check_pg_connectivity -check_std_cell_pins none

Verifies the macro-level power grid is fully connected to the supply ports without getting bogged down by checking millions of individual standard cell pins.

8.4.2 check_pg_missing_vias

Ensures that where vertical and horizontal power straps cross, a physical via successfully fuses them together.

8.4.3 check_pg_drc -do_not_check_in_hier_shapes_blocks

Checks the thick power metal routes for spacing and width Design Rule Check (DRC) violations, ensuring the foundry can manufacture the grid without shorts.

8.5 Automated script for Floorplan to PowerPlan

Rather than typing these steps interactively, the entire sequence from Chapter 4 through Chapter 8 can be executed deterministically via a master control script:

- source ./scripts/Floorplan_to_Powerplan/pd_flow_mv_macro_engine.tcl

This script handles the NDM creation, floorplan initialization, macro placement, port placement, UPF commitment, and the strict VDDL power grid generation in a single automated pass.

### Chapter 9: Congestion checks

Before committing to the heavy, time-consuming optimization phases, we must perform a "dry run" to ensure our floorplan, the 100 macro placements, and the multi-voltage areas won't cause a massive traffic jam of routing wires later on.

9.1 create_placement -floorplan

Instead of placing cells meticulously for timing, this command performs a fast, coarse placement of the ~76K standard cells. The -floorplan flag tells the tool to ignore heavy timing optimizations and simply drop the standard cells into the core (strictly respecting the 0.95V PD_TOP and 0.75V PD_LP voltage areas) to see how crowded the silicon canvas gets.

9.2 congestion reports

Once the cells are temporarily placed, we need to analyze if there is enough physical space (metal tracks) for all the logical connections to be wired together.

9.2.1 report_congestion -rerun_global_router

This command is critical. It triggers the global routing engine to do a quick, rough estimate of how the wires will be drawn.

- The -rerun_global_router switch forces the tool to calculate fresh routing data based exactly on the fast placement we just did.

- We review the log to identify "hotspots." If the tool reports high overflow (meaning more wires need to pass through a channel than there are physical metal tracks available, especially in the narrow channels between the 16 lp_ram_subsystem macros), the floorplan is unroutable. You must stop, adjust your macros or pin constraints, and try again.

9.3 reset_placement

Since create_placement -floorplan was only a temporary test to check for congestion hotspots, we do not want to keep it. Once we confirm the floorplan is highly routable, we execute reset_placement. This rips up the quick standard cell placement, leaving behind our clean, verified floorplan and power grids ready for formal placement.

### Chapter 10: Placement

Placement is the phase where the tool formally assigns a physical X,Y coordinate to all 76K standard cells. The goal is to optimize logic for setup timing, minimize total wire length, and strictly obey the 0.95V (PD_TOP) and 0.75V (PD_LP) voltage boundaries, all without creating routability issues.

10.1 Pre-requisite: Scandef Setup

Before executing the placement engine, we must load the scan chain definitions generated during synthesis. This ensures the tool knows how the flip-flops are chained together and can place them optimally to avoid routing congestion later.

- Command:

read_def ./outputs/fixed_high_fanouts/mv_macro_engine_top.scandef

10.2 Source mcmm_setup file - modes & scenarios

Before placement optimization begins, Multi-Corner Multi-Mode (MCMM) constraints must be loaded. The placement engine must "see" all operational scenarios simultaneously.

- Command:

source ./inputs/mcmm_files/mcmm_mv_macro_setup.tcl

(Note: Make sure to verify all inner paths, such as mcmm_corners_modes.tcl, to ensure the corner and mode definitions are accurately pointing to your .lib files).

For this mv_macro_engine design, it must balance the timing paths across different Process, Voltage, and Temperature (PVT) corners (e.g., worst-case timing at 0.85V for VDD and 0.65V for VDDL at 125°C, versus best-case at -40°C). Fixing a path in one corner without MCMM loaded can easily break it in another.

10.3 Source Placement Script

Instead of running commands manually, the placement engine (typically place_opt) is executed via this master script. It performs global placement, legalizes the cells onto the standard cell rows, and performs initial High-Effort logic optimization to fix setup timing violations.

- Command: source ./scripts/Placement_script/placement_mv_macro_engine.tcl

10.4 Sanity checks

10.4.1 check_legality

Validates that every single standard cell is placed legally. This means:

- Cells are aligned to the manufacturing site rows.

- Zero cells are overlapping.

- Zero 0.75V logic cells have accidentally leaked out of the PD_LP voltage area.

10.5 Reports

After placement, timing violations are expected, but physical Design Rule Checks (DRC) like transition and capacitance must be heavily scrutinized.

10.5.1 report_constraints -all_violators -max_transition -significant_digits 6

Identifies nets where the signal takes too long to toggle from 0 to 1 or 1 to 0 (slew rate). Slow transitions cause sluggish performance and massive short-circuit power dissipation.

10.5.2 report_constraints -all_violators -max_capacitance -significant_digits 6

Identifies cells trying to drive a load capacitance that exceeds the maximum limit characterized in the .lib files. This happens when a net is too long or a cell has too many fanouts.

10.6 Fixes for max trans

If the tool's auto-optimization leaves max transition violations, they must be resolved. Transition degradation is mostly caused by excessive RC wire delay.

10.6.1 Upsize cells (Not Recommended) - script: ./script/upsize_driver.tcl

Swaps the driving cell for a larger footprint cell with stronger drive strength. Not recommended manually because blindly upsizing steals area, increases power consumption, and often creates congestion hotspots in dense areas.

10.6.2 Insert Buffers (Not Recommended) - script: ./script/insert_buffer.tcl

Cuts a long, resistive wire in half by inserting a buffer to restore the signal slew. Not recommended manually at this stage because the clock tree doesn't exist yet; manual buffers can severely complicate CTS (Clock Tree Synthesis) later.

10.6.3 Magnet placement

A physical constraint technique. You define a "magnet" (often a macro or a critical level shifter) and the tool pulls all logically connected standard cells physically closer to it. Shorter wires mean less resistance, naturally fixing the transition violation.

10.6.4 Create_bounds

Forces a specific group of critical cells into a tightly restricted physical box. By caging the cells together, you prevent the placement engine from spreading them out across the die during optimization, controlling wire length.

10.6.5 Refine_placement

An incremental command run after applying bounds or magnets. It gently shifts surrounding cells to legalize the new constraints without destroying the overall placement optimization you already achieved.

10.6.6 Path grouping

Assigns a higher "weight" or priority to specific failing paths (for example, the data bus crossing from VDD_LP to VDD_DEFAULT). This forces the optimization engine to spend more computational effort fixing transitions on those specific nets at the expense of non-critical paths.

10.7 Fixes for max cap

Max capacitance violations are primarily driven by fanout (one cell trying to drive too many receivers).

10.7.1 Upsize cells (Not Recommended) - script: ./script/upsize_driver.tcl

Using a stronger gate to handle the large capacitive load. As before, this is a brute-force approach that degrades area and power metrics.

10.7.2 Insert Buffers (Not Recommended) - script: ./script/insert_buffer.tcl

Isolating the heavy load from the original driver by buffering the net.

10.7.3 Split load

Instead of one cell driving 40 endpoints, you insert parallel buffers. Buffer A drives 20 endpoints and Buffer B drives the other 20. This halves the capacitance seen by the drivers, fixing the DRC violation efficiently.

10.7.4 Cloning

A highly effective logic transformation. You instruct the tool to create an exact duplicate (clone) of the struggling logic cell. The tool then routes the inputs to both cells and distributes half the output fanout to the original cell and half to the clone. This perfectly resolves max cap issues on critical control signals without adding unnecessary buffer delays.

### Chapter 11: Clock Tree Synthesis

Up until this phase, the clock was "ideal" (zero delay, infinite drive strength). Clock Tree Synthesis (CTS) physically builds the clock distribution network, inserting buffers and routing wires to deliver the clock signal to all flip-flops simultaneously. In a multi-voltage design like the mv_macro_engine, clock domains crossing from the 0.95V (VDD_DEFAULT) domain to the 0.75V (VDD_LP) domain must be heavily scrutinized to minimize insertion delay differences caused by the voltage drop.

11.1 Type of CTS used - H-Tree

For this mv_macro_engine, we utilize an H-Tree Clock Routing topology via ICC2's multi-source clock subtree options. Instead of an unstructured tree, the clock is distributed symmetrically across the core through a highly balanced, repeating "H" pattern. This structural approach minimizes On-Chip Variation (OCV) and avoids top-level power planning metals by explicitly restricting root and internal clock routes to intermediate layers.

11.2 source ./scripts/CTS_H_TREE/cts_H_tree.tcl

This master script executes the complete clock building process for the standard cells. The execution flow is strictly controlled through the following stages:

- Lib Cell Purpose & Reference Setup: By default, tool cell assumptions are cleared. We explicitly source the allowed library references (source ./scripts/cts_include_refs.tcl or cts_include_libs.tcl) and promote specific standard high-drive buffers, inverters, and registers (NBUFF*LVT, INVX*_LVT, *DFF*) for clock tree usage.

- NDR Setup & Layer Constraints: Applies double-width and double-spacing rules (iccrm_clock_double_spacing) to the clock nets.

  - Sink Routing: Restricted to M4 and M5.

  - Root & Internal Routing: Restricted to M5 and M6. This intentionally preserves M7/M8 entirely for the robust multi-voltage power mesh.

- Clock Constraints & MCMM: Max transition is clamped at 0.15. Target skews are strictly corner-specific: 0.05ns for slow-slow (ss_125c) and a tighter 0.02ns for fast-fast (ff_m40c). Clock uncertainties are initialized at 0.1 for setup and 0.05 for hold.

- CRPR Validation: Clock Reconvergence Pessimism Removal (CRPR) is enabled (time.remove_clock_reconvergence_pessimism) to ensure the timing engine doesn't artificially penalize common clock path segments.

- Level Shifter Insertion: Exactly 2 level shifters are manually inserted on the clock nets crossing from the 0.95V main tree into the 0.75V VDD_LP low-power engine before building the tree. This ensures electrical integrity (preventing crowbar currents) and allows the CTS engine to "see" their inherent delay from the very beginning, building the rest of the 0.95V tree with matching delay buffers to balance the skew.

- CTS Execution Stages: The tree is built and routed incrementally using clock_opt -to build_clock and clock_opt -to route_clock.

- Post-CTS Convergence: Hold fixing cell options are isolated to high-threshold, low-leakage variants (DELLN*_HVT, NBUFF*_HVT). The tree is then optimized for high-effort hold timing and clock-aware scan reordering via clock_opt -from final_opto.

11.3 Issues / violations

Common violations post-CTS include:

- Max Skew: The difference in clock arrival time between the earliest and latest flip-flop is too large.

- Max Insertion Delay: The clock takes too long to travel from the port to the flip-flops, making it highly vulnerable to PVT variations.

- Pulse Width / Duty Cycle Distortion: The high/low shape of the clock wave degrades as it travels through unequal rise/fall buffer paths.

11.4 Fixes for Violations

To fix clock violations without ruining the tree, we apply specific CTS constraints:

- NDR Usage: Enforcing strict double-spacing rules on clock nets to shield them from neighbouring data lines.

- Restricted Cell Lists: Forcing the tool to use only specific, highly symmetrical clock buffers and inverters rather than standard logic buffers.

- Manual Guide Buffers: Manually dropping a strong clock buffer at a divergence point to force the tool to balance a stubborn branch.

11.5 Fixes for Hold / setup

Once the clock is "real," it takes time to reach the flip-flops, causing massive shifts in timing.

- Setup Fixes: Setup paths fail if data arrives too late. The tool fixes this by downsizing data path logic, cloning heavily loaded cells, or utilizing Useful Skew.

- Hold Fixes: Hold paths fail if data arrives too fast (racing past the clock). The tool exclusively fixes this by inserting delay buffers (like the DELLN*_HVT cells defined in the script) into the data path to slow the signal down.

11.6 Sanity checks

Verify the clock tree structure by ensuring there are no unrouted clock nets, no overlapping clock buffers, and that the clock signal cleanly passes through the newly inserted level shifters when crossing into the 0.75V domain.

11.7 Reports

The script automatically dumps comprehensive summaries into the ./report/ directory:

- report_clock_tree -summary -> Analyzes the longest and shortest paths.

- report_timing -delay_type max/min -> Validates post-CTS setup and hold timing.

- report_clock_qor -type <option> -> A comprehensive command to check the Quality of Results for the clock network. To deeply analyze the clock structure, you can specify the following types:

  - area

  - latency

  - power

  - summary

  - balance_groups

  - level

  - robustness

  - timing_aware_local_skew

  - drc_violators

  - local_skew

  - structure

### Chapter 12: Routing

With the standard cells placed and the clock tree built, the remaining task is to wire all the logic together. The routing engine must physically connect millions of pins using the available metal layers while avoiding congestion, shorts, and physical manufacturing rules.

12.1 source ./scripts/Routing/routing_script.tcl

Detailed routing is executed via this master script, which performs Production Routing & Signoff Optimization. It operates in three distinct phases:

- Global Routing: Divides the chip into grids and plans the general path for every wire, managing overall congestion.

- Track Assignment: Assigns those general paths to specific metal tracks (e.g., Metal 3, Metal 4) in the routing channels.

- Detailed Routing: Connects the actual standard cell pins, navigating around obstacles, vias, and adjacent wires to resolve all physical DRCs.

The script enforces critical optimizations:

- Timing-Driven Routing: Set to true for global, track, and detail stages to ensure the router honors the setup timing constraints defined in the SDC.

- Crosstalk Aware Routing: Enabled for global and track stages to minimize noise injection between adjacent parallel wires.

- SI Analysis: Enables Signal Integrity (SI) analysis with statistical crosstalk aggregation (route.time.si_enable_analysis).

12.2 Sanity checks

12.2.1 check_design -checks pre_route_stage

Before the heavy detailed routing begins, this crucial check ensures the database is clean. It verifies that placement legality holds, the multi-voltage power grid (VDD_DEFAULT and VDD_LP) is fully intact, and the clock tree is fully routed and fixed (set_dont_touch_network -clock_only [get_ports *clk*]).

12.2.2 check_lvs -max_errors 0

Performs Layout Versus Schematic verification to ensure the physical metal connectivity perfectly matches the logical netlist, ensuring zero net-mismatch errors.

12.2.3 check_routes

Validates the integrity of the routed database, identifying any incomplete connections or illegal wiring topologies.

12.2.4 check_pg_drc -do_not_check_in_hier_shapes_blocks

Checks the thick power metal routes for spacing and width Design Rule Check (DRC) violations, while ignoring blocks to focus strictly on the grid.

12.3 Reports

12.3.1 check_routability

This command analyzes the routing grids for DRC hotspots. It ensures that the router did not leave open nets (unrouted wires) or short circuits (wires touching) that cannot be resolved in subsequent optimization passes.

12.4 Antenna Rule

During plasma etching in manufacturing, long metal wires act like antennas, accumulating static electrical charge. If this charge has nowhere to go, it blows out the delicate thin-oxide gate of the transistor it connects to.

We enforce these protections by sourcing the foundry-provided ruleset:

- Command: source ./scripts/Routing/saed32nm_ant_1p9m.tcl

The router detects and fixes Antenna Violations using two automated methods:

- Layer Hopping: Breaking the long wire and routing it up to the highest metal layer to interrupt charge accumulation.

- Antenna Diodes: Inserting a reverse-biased diode near the transistor gate to bleed the charge safely.

12.5 Optimization & Save

Once routing is complete, we run route_opt to perform final logic-gate optimization and wire-length refinement. The design is then finalized and saved to the database:

- Command: save_block -as route_opt_all

### Chapter 13: Signoff

Signoff is the final phase of the ASIC design cycle. The physical database is locked and rigorous checks are performed using specialized signoff tools to guarantee the chip will function correctly after manufacturing.

13.1 source ./scripts/signoff_script.tcl

The signoff process involves exporting the physical design data for final verification across several domains:

- Parasitic Extraction (StarRC): Extract SPEF (Standard Parasitic Exchange Format) using StarRC. This tool extracts the final, highly accurate resistance and capacitance values (RC) of every single routed wire in the 32nm design.

- Static Timing Analysis (PrimeTime): Uses the extracted SPEF data to run a final, golden timing check across all MCMM corners. During this phase, we generate images (visual bottleneck histograms and graphical path profiles) using PrimeTime to visually document timing closure. The mv_macro_engine design must have zero setup and zero hold violations across both the 0.95V (VDD_DEFAULT) and 0.75V (VDD_LP) domains.

- Physical Verification (IC Validator / Calibre):

  - DRC (Design Rule Check): Ensures spacing, width, and via rules strictly meet the 32nm foundry manual.

  - LVS (Layout Versus Schematic): Extracts the physical shapes back into a netlist and compares it against the golden synthesis netlist to ensure zero missing or shorted connections.

- IR Drop / EM Signoff (RedHawk / Voltus): Confirms the power mesh—especially the 0.75V VDD_LP grid connected to the SRAM macro guard bands—delivers adequate voltage to all instances under peak switching activity without electromigration failure.

13.2 Tapeout

Once all signoff checks report zero violations, the design achieves Tapeout Readiness. The final physical shapes are exported as a GDSII stream format file and sent to the foundry for mask creation and silicon fabrication.

### Chapter 14: Sign-off Timing Analysis & ECO Flow

Phase 1: ICC2 Preparation & Data Export

Before moving to timing sign-off, the physical database must be structurally sound and free of manual routing artifacts.

- Final Physical Verification: Ensure the block is physically clean after any manual GUI routing fixes.

check_pg_drc -do_not_check_in_hier_blocks

check_lvs -max_errors 0

check_routes

- Export the Post-Route Netlist:

write_verilog ./PRIMETIME/inputs/routed_mv_macro_netlist.v

- Export Clean Post-Route SDCs: Export the constraints directly from the routed database to capture the true propagated clock structures. Do not reuse pre-route SDCs.

write_sdc -scenarios func.ss_125c -output ./PRIMETIME/inputs/ss_125_clean.sdc

write_sdc -scenarios func.ff_m40c -output ./PRIMETIME/inputs/ff_m40_clean.sdc

Phase 2: Parasitic Extraction (StarRC)

- Ensure the directory structure is set up:

mkdir -p STARRC/{inputs,outputs/{spef,work},scripts}

- Execute the StarRC extraction scripts.

- Copy the highly accurate, 3D-extracted .spef files (cbest.spef and cworst.spef) to the ./PRIMETIME/inputs/ directory.

Phase 3: PrimeTime Environment Setup & Single-Scenario Checks

Setting up the PrimeTime scripts correctly is critical to avoid multi-voltage scaling crashes (SLG-303) and unresolved link references (LNK-003).

- Configure link_library Properly:

  - Rule: The link_library must only contain the primary/default voltage base cells (0.95V), the SRAM, and the necessary level-shifter libraries (both ulvl and dlvl for all VT flavors: LVT, RVT, HVT).

  - Do NOT include the secondary voltage base cells (e.g., 0.75V or 0.85V) in the link_library, or the scaling groups will crash.

- Configure Scaling Groups: Define the scaling groups to map the primary 0.95V cells to their respective secondary voltage (0.75V for max, 0.85V for min) variants.

- Clean SDC Sourcing: Source the _clean.sdc files exported from ICC2. Remove any manual hacks like set_propagated_clock or remove_ideal_network from the PT scripts, as the post-route SDC handles this natively.

- Validation: Invoke pt_shell, source func_max.tcl, and verify report_global_timing. Exit the shell completely, restart it, and repeat for func_min.tcl.

Phase 4: DMSA (Distributed Multi-Scenario Analysis) Sign-off

Once individual scenarios link cleanly, use a Master Script to run them concurrently and generate a unified ECO file.

- Launch DMSA:

pt_shell -multi_scenario

- Execute the Master Manager Script: This script should allocate cores, create scenarios, source the verified func_max and func_min scripts to the background workers, and perform a unified timing check.

- Generate the ICC2 ECO File:

write_changes -format icc2tcl -output ./outputs/Multi_scenario_final_eco.tcl

Phase 5: ICC2 ECO Implementation

If PrimeTime generates physical changes (like cell sizing or buffer insertion), implement them precisely without disrupting the rest of the layout.

- Source the DMSA ECO file in icc2_shell:

source ./PRIMETIME/outputs/Multi_scenario_final_eco.tcl

- Legalize and Route the ECOs safely:

check_legality

legalize_placement -incremental -movable_distance 50

connect_pg_net

check_legality

route_eco -utilize_dangling_wires true reuse_existing_global_route true

-reroute modified_nets_first_then_others

- Run the Final Verification Checklist:

check_lvs -max_errors 0

check_routes

check_pg_drc -do_not_check_in_hier_blocks

save_block -as route_signoff_pristine

Phase 6: Sign-off Realities & Waivers (Golden Rules)

PrimeTime is the ultimate source of truth. Rely on its SPEF-driven physics over ICC2's pessimistic routing estimates.

- The 5-Femtosecond Hold Reality: If PT reports a minuscule hold violation (e.g., ~0.000005 ns), do not attempt to fix it. The library lacks an X1 drive strength cell to add such a tiny delay, and inserting an X2 buffer (intrinsic delay ~15-20ps) will immediately create a setup violation. Waive this as mathematical noise within OCV margins.

- The Pessimism Trap: If ICC2 reports dynamic Max Capacitance, Max Transition, or minor Setup/Hold violations, but PrimeTime reports 0 across the board using SPEF extraction, PrimeTime wins. Do not run further optimizations (route_opt) in ICC2 to chase false violations. Write a waiver and proceed to tape-out.

### Chapter 15: Advanced IR Drop

15.1 Power Integrity & Dynamic IR-Drop Signoff

Even if Static Timing Analysis reports zero violations, the silicon can still fail if the power delivery network (PDN) collapses under heavy computational load. Because modern ICC2 environments rely on the Ansys RedHawk-SC fusion integration for native analyze_rail commands, missing this integration requires Power Integrity (PI) analysis to be performed as an external signoff step using dedicated standalone tools (e.g., PrimePower, Voltus, or standalone RedHawk).

15.2 Introduction & Physics/Theory

When thousands of logic gates in the mv_macro_engine toggle simultaneously, they pull a massive surge of current through the resistive metal tracks of the PDN. According to Ohm’s Law (V=I×R), this current draw causes the local supply voltage to sag. If the 0.75V VDDL supply temporarily drops to 0.68V, the standard cells switch slower, potentially violating setup time limits that were verified under ideal 0.75V conditions.

15.3 Exporting the Design for External Analysis

To run power analysis outside of ICC2, the physical and parasitic data must be accurately exported. The external PI tool requires a snapshot of the fully routed design, the parasitic resistances of the power grid, and the switching activity.

# Export the physical layout

write_def -version 5.8 -routing -all_blocks reports/mv_macro_engine_top.def

# Export the parasitic RC data

write_parasitics -format SPEF -output reports/mv_macro_engine_top.spef

# Optional: Generate a power report in ICC2 for baseline estimation

report_power -scenarios [current_scenario] > reports/icc2_baseline_power.rpt

15.4 Files / Inputs Required for Signoff

The external Power Integrity team will ingest the following files:

- DEF (.def): Contains the physical placement of all standard cells, macros, and the exact metal routing of the power grid.

- SPEF (.spef): Contains the exact resistance and capacitance values of the metal routes.

- .lib (Liberty): Contains the internal power models and current draw characteristics for the 0.95V and 0.75V standard cells.

- VCD/FSDB: A Value Change Dump generated by the verification team running a gate-level simulation of an active encryption payload. This provides the exact switching timestamp of every net for dynamic analysis.

15.5 Validating the VDDL Topology

This standalone analysis proves the physical viability of your custom power plan. The PI tool specifically evaluates the VDDL low-power region to ensure the power mesh connecting directly to the inner guard band is delivering uncompromised current. If the manual topology (which intentionally bypassed macro ring automation) is too resistive, the tool will flag massive voltage droop hotspots in the center of the engine.

15.6 Heatmaps & Signoff Reports

The external tool will generate two primary deliverables:

- Dynamic Voltage Drop Heatmap: A visual color map of the die highlighting areas where the voltage drops below the accepted threshold (e.g., a drop greater than 5%).

- Electromigration (EM) Reports: Identifies metal tracks or vias where the current density (J) exceeds foundry limits, which could cause the wire to degrade or melt over time.

15.7 Fixes for IR-Drop Violations

If the PI team flags unacceptable voltage drops, fixes must be implemented back in the ICC2 environment via Engineering Change Orders (ECOs):

- Adding Decap Cells: Decoupling capacitors act like tiny local batteries. They are inserted into empty spaces near hotspots to supply instantaneous charge during switching bursts.

- Thickening Local Straps: Manually dropping additional power vias or widening specific M7/M8 straps over the starving region to lower the resistance (R).

- Spreading the Logic: Applying density padding constraints in ICC2 and incrementally refining placement to break up dense clusters of high-drive standard cells.

### Chapter 16: Hard Macro Conversion (Block Abstraction)

Once the mv_macro_engine_top core has achieved timing closure, clean IR-drop, and zero DRC/LVS violations, it can be "hardened." Hardening means sealing the internal ~76K standard cells and 100 macros, converting the entire block into a single, black-box component (a Hard Macro) to be placed inside a larger System-on-Chip (SoC).

The top-level SoC tools do not need to see your internal gates; they only need to know the physical boundaries, the pin locations, the power rails, and the timing of the input/output ports.

16.1 Physical Abstraction (Creating the NDM / FRAM View)

To use the macro in a top-level ICC2 flow, we must generate a lightweight physical abstraction. This view hides the internal standard cells and only exposes the core boundary, port geometries, and routing blockages (to prevent the top-level router from accidentally dropping vias into your internal logic).

*** Example:

# Open the fully routed and signed-off block

open_block mv_macro_engine_top_routed

# Create the abstract view (Frame) for top-level hierarchical design

create_abstract -read_only

This command generates an abstract NDM view. It identifies which metal layers are saturated by the core and creates placement/routing blockages, leaving only the highest empty metal layers available for top-level "over-the-cell" routing.

16.2 Timing Abstraction (Extracted Timing Model - ETM)

Loading internal gates into the top-level Static Timing Analysis (STA) would crash the tool or take days to run. Instead, we generate an Extracted Timing Model (ETM), which reduces the entire block's timing to just the setup/hold requirements of the input pins and the clock-to-q delays of the output pins.

This is typically done in PrimeTime:

# Inside PrimeTime

read_verilog mv_macro_engine_top_routed.v

read_parasitics mv_macro_engine_top.spef

# ... load constraints and link ...

# Extract the timing model to generate a liberty file

extract_model -format {lib db} -output mv_macro_engine_top_macro

This creates mv_macro_engine_top_macro.lib and .db. The SoC synthesis and timing teams will use these exactly like they use standard cell libraries.

16.3 Multi-Voltage & Power Pin Promotion

Because this is a dual-voltage macro, the top-level SoC must provide both 0.95V and 0.75V to your block. During the abstraction process, your internal power meshes must be promoted as logical Supply Ports.

- The top-level floorplanner will look at your .lef or NDM abstract to find the exact X,Y coordinates of your VDDL guard band mesh.

- The top-level Power Delivery Network (PDN) will then drop massive power vias directly onto those exposed geometries to feed your computational engine.

16.4 Exporting Industry-Standard Handoff Files

If the SoC is being assembled using third-party tools (not strictly ICC2), you must export standard interchange formats rather than just an NDM. A complete "Hard Macro IP Kit" consists of four files:

# 1. GDSII: The complete, flattened physical layout for manufacturing

write_gds -design mv_macro_engine_top_routed ./export/mv_macro_engine_top.gds

# 2. LEF (Library Exchange Format): The physical boundary and pin locations

write_lef ./export/mv_macro_engine_top.lef

# 3. Verilog (Black-box): A structural shell for top-level LVS

# (Contains only the module declaration and port list, no internal logic)

write_verilog -compress false -hierarchy abstract ./export/mv_macro_engine_top_bb.v

The Final Hard Macro Deliverables:

- mv_macro_engine_top.gds (Manufacturing shapes)

- mv_macro_engine_top.lef (Physical pins and blockages for the top-level router)

- mv_macro_engine_top.lib / .db (Timing models for the top-level STA)

- mv_macro_engine_top_bb.v (Empty wrapper for LVS)

- mv_macro_engine_top.upf (The power intent, declaring the dual VDD/VDDL pins)

### Chapter 17: Formal Verification (Logic Equivalence Checking)

Even a single flipped gate during placement optimization or a botched routing connection can completely break the functionality of the mv_macro_engine. Formal Verification uses mathematical proofs to guarantee that the final, routed netlist (with all its inserted clock buffers, level shifters, and scan chains) is 100% logically identical to your original Verilog RTL.

Important Tool Requirement: Formal verification cannot be executed inside the icc2_shell. It requires a dedicated external equivalence checking tool like Synopsys Formality or Cadence Conformal.

16.1 Execution via Synopsys Formality

The flow compares a "Reference" (your golden RTL) against an "Implementation" (your routed netlist).

Tcl (Formality Script):

Tcl

# 1. Setup Reference (Golden RTL)

read_sverilog -container r {./inputs/sram_stub.v ./inputs/mv_macro_engine_top_sfo.v}

read_db -container r {./inputs/libs/saed32_lv.db ./inputs/libs/saed32_hv.db}

set_top r:/WORK/mv_macro_engine_top

# 2. Setup Implementation (Routed Netlist from ICC2)

read_verilog -container i ./export/mv_macro_engine_top_bb.v

read_db -container i {./inputs/libs/saed32_lv.db ./inputs/libs/saed32_hv.db}

read_upf -container i ./outputs/mv_macro_engine_top.upf

set_top i:/WORK/mv_macro_engine_top

# 3. Match and Verify

match

verify

### Chapter 18: Design for Manufacturing (DFM)

Just because a design routes cleanly and passes timing does not mean the foundry can manufacture it reliably. DFM techniques are applied in ICC2 just before exporting the final GDSII to maximize silicon yield and prevent microscopic manufacturing defects.

18.1 Metal Fill (Dummy Fill)

During manufacturing, chemical-mechanical polishing (CMP) relies on a uniform density of metal across the die. If there are large empty spaces between the routing tracks, the polishing pad will "dish" into the silicon, destroying adjacent wires. We insert electrically floating "dummy" metal shapes to ensure uniform density.

Important Tool Requirement: Running native metal fill (signoff_create_metal_fill) inside ICC2 strictly requires the IC Validator executable and fusion integration to be present in your environment. If your environment lacks IC Validator, this step is typically deferred and handled by the backend CAD team using standalone signoff tools.

18.2 Redundant Via Insertion (Via Optimization)

A single via connecting Metal 3 to Metal 4 can easily fail during fabrication due to dust or misalignment. Wherever there is physical space, we instruct ICC2 to drop a second, redundant via next to the first one. If one fails, the other keeps the connection alive.

Unlike metal fill, this can be executed natively in ICC2 without external validator tools.

Tcl (ICC2):

Tcl

# Enable extra via insertion prioritizing a low-to-high layer order

set_app_options -name route.detail.insert_redundant_vias_layer_order_low_to_high -value true

# Run incremental route optimization to apply the redundant vias safely

route_opt -incremental

### Chapter 19: The ECO Flow & Final Handoff

What happens if the verification team finds a critical bug in the RTL after you have finished Signoff? You cannot afford to rerun the entire multi-day flow from Chapter 4. Instead, we use the Engineering Change Order (ECO) flow to freeze the silicon, freeze the routing, and surgically swap out or insert a few specific gates and patch the local routing without destroying your timing closure.

19.1 Functional Metal-Only ECO (Pre-Mask)

If a logic bug is found late, we try to fix it using "spare cells" (floating, unused logic gates intentionally scattered across the floorplan earlier in the flow). By only changing the metal routing to connect a spare cell and leaving the base silicon layers untouched, we save massive mask remanufacturing costs.

Tcl (ICC2):

Tcl

# Swap connections to an available spare cell

disconnect_net [get_nets broken_logic_net] [get_pins bad_cell/Y]

connect_net [get_nets broken_logic_net] [get_pins spare_inv/Y]

# Reroute ONLY the patched connections

route_eco

19.2 Final GDSII Handoff

Once all ECOs are applied, DFM is complete, and the design is fully verified, the physical shapes are exported as a GDSII stream format file. This is the final deliverable sent to the foundry for mask creation and silicon fabrication.

Tcl (ICC2):

Tcl

# Export the final flattened layout for manufacturing

write_gds -design mv_macro_engine_top_routed ./export/mv_macro_engine_top.gds